| EXERCISE NO:1 DATE:09/01/2025 | RGB TO GRAY HISTOGRAM EQUILIZATION IMAGE RESIZING |
| --- | --- |

AIM:

To view the rgb image to grayscale image and histogram equilization of the grayscale image and the image resizing

LENA IMAGE

ALGORITHM:

- Importing the library function of the open cv for image processing

- Read the image using the imread function

- Convert the image using function of the cvtColor  in BGR to RGB

- Execute the image

- After the  image is convert into the GRAY color using the function cvt Color in RGB to GRAY

- Manuval code of the RGB to gray scale

- Histogram equilization and using the RAVEL function and calculating the histogram using the CALC HIST and also the manuval code of the histogram equilization using the table

- After that the resizing the image using the function resize

CODE:

import cv2

import numpy as np

import matplotlib.pyplot as plt

img1 = cv2.imread(r"lena.png")

img3 = cv2.cvtColor(img1,cv2.COLOR_BGR2RGB)

img_2 = cv2.cvtColor(img3,cv2.COLOR_RGB2GRAY)

plt.figure(figsize=(6,5))

plt.subplot(121)

plt.title("lena in rgb")

plt.imshow(img3)

OUTPUT:

CODE:

import cv2

import numpy as np

import matplotlib.pyplot as plt

plt.figure(figsize=(4,2))

plt.subplot(121)

plt.title("grayimage")

plt.imshow(img_2 , cmap='gray')

red,green,blue = img1[:,:,0],img1[:,:,1],img1[:,:,2]

gamma=1

gray=0.299*red**gamma+0.587*green**gamma+0.114*blue**gamma

print(gray)

plt.figure(figsize=(4,5))

plt.subplot(121)

plt.imshow(gray,cmap='gray')

OUTPUT:

f

The grayscale image output

The manuval code of the the grayscale image

HISTOGRAM EQUALIZATION PLOT USING THE LIBRARIES AND THE MANUVAL

CODE:

import cv2

import numpy as np

import matplotlib.pyplot as plt

img_hist=cv2.calcHist([img1],[0],None,[256],[0,256])

plt.subplot(222)

plt.title("histogram")

plt.plot(img_hist)

final_image =cv2.equalizeHist(img_2)

res=np.hstack((gray,final_image))

plt.imshow(res,cmap='gray')

histg = cv2.calcHist([img_2],[0],None,[256],[0,256])

plt.plot(histg)

plt.hist(img_2.ravel(),256,[0,256])

plt.hist(img1.ravel(),256,[0,256])

MANUVAL CODE:

def manual_histogram_equalization(image):

if len(image.shape) == 3:

image = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)

hist, bins = np.histogram(image.flatten(), 256, [0, 256])

cdf = hist.cumsum()

cdf_normalized = cdf * hist.max() / cdf.max()

cdf_m = np.ma.masked_equal(cdf, 0)

cdf_m = (cdf_m - cdf_m.min()) * 255 / (cdf_m.max() - cdf_m.min())

cdf = np.ma.filled(cdf_m, 0).astype('uint8')

img_equalized = cdf[image]  #

return img_equalized

img1 = cv2.imread(r"lena.png")

if img1 is None:

print("Error: Image not loaded. Check the file path.")

else:

img_equalized = manual_histogram_equalization(img1)

plt.figure(figsize=(10, 5))

# Original Image

plt.subplot(1, 2, 1)

plt.imshow(cv2.cvtColor(img1, cv2.COLOR_BGR2RGB))

plt.title('Original Image')

plt.axis('off')

# Equalized Image

plt.subplot(1, 2, 2)

plt.imshow(cv2.cvtColor(img_equalized, cv2.COLOR_BGR2RGB))

plt.title('Histogram Equalized Image')

plt.axis('off')

plt.show()\

HISTOGRAM  CODE:

import cv2

import numpy as np

import matplotlib.pyplot as plt

img = cv2.imread(r"lena.png")

if img is None:

print("Error: Image not loaded. Check the file path.")

else:

gray_img = cv2.cvtColor(img, cv2.COLOR_BGR2GRAY)

hist = cv2.calcHist([gray_img], [0], None, [256], [0, 256])

plt.figure(figsize=(10, 5))

plt.title('Histogram of Grayscale Image')

plt.xlabel('Pixel Intensity')

plt.ylabel('Frequency')

plt.plot(hist)

plt.xlim([0, 256])

plt.show()

OUTPUT:

Using libraries output

Histogram equalize output

Histogram  output  of example 1 :

Histogram equalize output  of manuval  :

RESIZING THE IMAGE

CODE:

import cv2

import numpy as np

import matplotlib.pyplot as plt

resized_image = cv2.resize(img_2, (1000, 1000))

plt.imshow(img_2,cmap='grey')

plt.figure(figsize=(10,10))

plt.subplot(121)

plt.imshow(resized_image,cmap='grey')

plt.title("resize image")

OUTPUT:

SUNG IMAGE

ALGORITHM:

- Importing the library function of the open cv for image processing

- Read the image using the imread function

- Convert the image using function of the cvtColor  in BGR to RGB

- Execute the image

- After the  image is convert into the GRAY color using the function cvt Color in RGB to GRAY

- Manuval code of the RGB to gray scale

- Histogram equilization and using the RAVEL function and calculating the histogram using the CALC HIST and also the manuval code of the histogram equilization using the table

- After that the resizing the image using the function resize

CODE:

import cv2

import numpy as np

import matplotlib.pyplot as plt

img1 = cv2.imread(r"SUNG.png")

img3 = cv2.cvtColor(img1,cv2.COLOR_BGR2RGB)

img_2 = cv2.cvtColor(img3,cv2.COLOR_RGB2GRAY)

plt.figure(figsize=(6,5))

plt.subplot(121)

plt.title("lena in rgb")

plt.imshow(img3)

OUTPUT:

CODE:

import cv2

import numpy as np

import matplotlib.pyplot as plt

plt.figure(figsize=(4,2))

plt.subplot(121)

plt.title("grayimage")

plt.imshow(img_2 , cmap='gray')

red,green,blue = img1[:,:,0],img1[:,:,1],img1[:,:,2]

gamma=1

gray=0.299*red**gamma+0.587*green**gamma+0.114*blue**gamma

print(gray)

plt.figure(figsize=(4,5))

plt.subplot(121)

plt.imshow(gray,cmap='gray')

OUTPUT:

The grayscale image output

The manuval code of the the grayscale image

HISTOGRAM EQUALIZATION PLOT USING THE LIBRARIES AND THE MANUVAL

CODE:

import cv2

import numpy as np

import matplotlib.pyplot as plt

img_hist=cv2.calcHist([img1],[0],None,[256],[0,256])

plt.subplot(222)

plt.title("histogram")

plt.plot(img_hist)

final_image =cv2.equalizeHist(img_2)

res=np.hstack((gray,final_image))

plt.imshow(res,cmap='gray')

histg = cv2.calcHist([img_2],[0],None,[256],[0,256])

plt.plot(histg)

plt.hist(img_2.ravel(),256,[0,256])

plt.hist(img1.ravel(),256,[0,256])

MANUVAL CODE:

def manual_histogram_equalization(image):

if len(image.shape) == 3:

image = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)

hist, bins = np.histogram(image.flatten(), 256, [0, 256])

cdf = hist.cumsum()

cdf_normalized = cdf * hist.max() / cdf.max()

cdf_m = np.ma.masked_equal(cdf, 0)

cdf_m = (cdf_m - cdf_m.min()) * 255 / (cdf_m.max() - cdf_m.min())

cdf = np.ma.filled(cdf_m, 0).astype('uint8')

img_equalized = cdf[image]  #

return img_equalized

img1 = cv2.imread(r"SUNG.png")

if img1 is None:

print("Error: Image not loaded. Check the file path.")

else:

img_equalized = manual_histogram_equalization(img1)

plt.figure(figsize=(10, 5))

# Original Image

plt.subplot(1, 2, 1)

plt.imshow(cv2.cvtColor(img1, cv2.COLOR_BGR2RGB))

plt.title('Original Image')

plt.axis('off')

# Equalized Image

plt.subplot(1, 2, 2)

plt.imshow(cv2.cvtColor(img_equalized, cv2.COLOR_BGR2RGB))

plt.title('Histogram Equalized Image')

plt.axis('off')

plt.show()\

HISTOGRAM  CODE:

import cv2

import numpy as np

import matplotlib.pyplot as plt

img = cv2.imread(r"SUNG.png")

if img is None:

print("Error: Image not loaded. Check the file path.")

else:

gray_img = cv2.cvtColor(img, cv2.COLOR_BGR2GRAY)

hist = cv2.calcHist([gray_img], [0], None, [256], [0, 256])

plt.figure(figsize=(10, 5))

plt.title('Histogram of Grayscale Image')

plt.xlabel('Pixel Intensity')

plt.ylabel('Frequency')

plt.plot(hist)

plt.xlim([0, 256])

plt.show()

OUTPUT:

Using libraries output

Histogram equalize output

Histogram  output  of example 1 :

Histogram equalize output  of manuval  :

RESIZING THE IMAGE

CODE:

import cv2

import numpy as np

import matplotlib.pyplot as plt

resized_image = cv2.resize(img_2, (3000, 5000))

plt.imshow(img_2,cmap='grey')

plt.figure(figsize=(10,10))

plt.subplot(121)

plt.imshow(resized_image,cmap='grey')

plt.title("resize image")

OUTPUT:

RESULT:

We  have successfully executed  basic image and functions to reize image histogram equilization  and RGB to GRAY for lena and sung image